✅ Hardware-in-the-Loop (HIL) Test Requirements for GPS Conversion & Simulation Block

HIL-RQ1: System Initialization

- Objective: Verify correct initialization of start positions and offset parameters.

- Test Method:

  - Load known values for Const_Offset_Lat, Const_Offset_Long, Startposition_Latitude, and Startposition_Longtide.

  - Observe the first Veh_Pos_Lati and Veh_Pos_Longi outputs.

- Pass Criteria: Output GPS coordinates match expected initial values within ±0.00001°.

HIL-RQ2: Cartesian-to-GPS Conversion Validation

- Objective: Validate real-time conversion of Pos_x and Pos_y to Latitude and Longitude.

- Test Method:

  - Simulate real-time movement (e.g., driving trajectory) with known Cartesian values.

  - Feed values to Pos_x, Pos_y via HIL interface.

  - Compare system outputs to offline MATLAB-calculated GPS values.

- Pass Criteria: Deviation < ±0.0001° for both latitude and longitude.

HIL-RQ3: Simulation Toggle Functionality

- Objective: Ensure proper switching between real and Nürburgring simulation GPS data.

- Test Method:

  - Dynamically change Racetrack_Simulation signal during run.

  - Monitor Veh_Pos_Lati and Veh_Pos_Longi for expected switching behavior.

- Pass Criteria: No glitch/discontinuity; values reflect change in source instantly.

HIL-RQ4: Race Track Data Playback Verification

- Objective: Validate simulation playback of Nürburgring data stream.

- Test Method:

  - Enable Racetrack_Simulation and compare output stream to reference trace data.

- Pass Criteria: GPS values follow known Nürburgring profile within ±0.00005°.

HIL-RQ5: Real-Time Execution Compliance

- Objective: Ensure that GPS conversion operates within timing constraints.

- Test Method:

  - Use HIL system profiler to log task execution time per conversion cycle.

- Pass Criteria: Each cycle completes in ≤10 ms.

HIL-RQ6: Boundary Condition Test

- Objective: Verify stability near coordinate extremes or large input values.

- Test Method:

  - Inject edge-case inputs: Pos_x = ±1e6, Pos_y = ±1e6.

- Pass Criteria: System does not crash or produce NaN/Inf; outputs saturate safely.

HIL-RQ7: Signal Dropout/Noise Resilience

- Objective: Test behavior when Pos_x or Pos_y drop out or contain noise.

- Test Method:

  - Inject brief NaN pulses or high-frequency noise into inputs.

- Pass Criteria: GPS output recovers to valid state within 1 cycle; optional smoothing.

HIL-RQ8: Fault Injection - Parameter Corruption

- Objective: Assess fault handling when offset constants are corrupted (e.g., overflow or NaN).

- Test Method:

  - Force constants (Const_Offset_*) to invalid values via HIL API.

- Pass Criteria: GPS output defaults to fallback or freezes with diagnostic flag.

HIL-RQ9: Latency Between Input and Output

- Objective: Measure propagation delay from Cartesian input to GPS output.

- Test Method:

  - Inject step signal into Pos_x and measure delay to Veh_Pos_Longi change.

- Pass Criteria: Output latency ≤10 ms.

HIL-RQ10: Continuous 1-Hour Run Stability

- Objective: Validate system stability under prolonged use.

- Test Method:

  - Run GPS conversion for 1 hour with dynamic position inputs.

- Pass Criteria: No memory leak, stack overflow, or signal loss observed.